# Overview

This document contains the necessary information to build the MMC60 Assistive Switch. The MMC60 is an assistive switch with a 60 mm diameter top. It is meant to be a robust switch with an integrated ¼-20 mounting option for users who require a large, robust switch.

Contents

Overview	1

Maker Checklist	3

Maker To Do List	3

Items to Give to User	3

Tool List	4

Tools / Equipment	4

Supplies	4

Personal Protective Equipment (PPE)	4

Customization Guide	5

3D Printing Guide	6

3D Printing Summary	6

3D Printing Settings	6

Post-Processing	6

Examples of Quality Prints	6

Maker Component List	8

Assembly Guide	8

Part A: Required Components	8

Part A: Required Tools and Supplies	8

Part A: Required Personal Protective Equipment (PPE)	8

Testing	8

Troubleshooting	8

# Maker Checklist

This list provides an overview of the steps required to build and deliver the MMC60 Assistive Switch.

## Maker To Do List

- Read through the Maker Guide to become familiar with required components, tools, supplies, safety gear, and overall assembly steps.

- Talk to the User about customization options

  - Colour of the MMC60 base bottom and button top

  - Which button top they would like

    - The standard button top is a flat disk on top.

    - You can send the Custom Top Guide to the user to choose custom top from.

  - How they would like to receive the “User Guide” (PDF or physical copy)

- Order hardware components

  - Please see the Bill of Materials (BOM) for links to places to purchase the components

- Gather tools, supplies, and safety equipment.

- Assemble the MMC60 Assistive Switch

- Test the MMC60 Assistive Switch

- Print “User Guide” (if the User would like a physical copy)

## Items to Give to User

- Completed MMC60 Assistive Switch

- 3.5mm mono cable

- “User Guide”

# Tool List

## Tools / Equipment

| Tool ID | Description | Required / Recommended | Notes |
| --- | --- | --- | --- |
| T01 | Wire cutter/stripper | Required | Cutting wires to length and stripping insulation from wires. |
| T02 | Soldering iron | Required | Soldering wires to switch and jack. |
| T03 | Phillips Head screwdriver | Required | Tightening screws. |
| T04 | Adjustable wrench or wrench/ratchet that fits hex bolt | Recommended | Seating the T-nut in the button base bottom correctly. |

## Supplies

| Supplies ID | Description | Quantity |  | Notes |
| --- | --- | --- | --- | --- |
| S01 | Solder | Small amount | Soldering wires to switch and jack. |  |

## Personal Protective Equipment (PPE)

| PPE ID | Description | Notes |
| --- | --- | --- |
| P01 | Safety glasses/goggles | We recommend using eye protection while assembling the switch. |

# Customization Guide

The device can be printed in the user’s desired colour.

We have also designed ten custom switch tops, which are shown in the table below.

| MMC60 Assistive Switch Custom Tops |  |  |
| --- | --- | --- |
| MMC60_Button_Cups.stl | MMC60_Button_Hex4.stl | MMC60_Button_Hex8.stl |
| MMC60_Button_Rings.stl | MMC60_Button_Rings2.stl | MMC60_Button_Spiral.stl |
| MMC60_Button_Gaming_A.stl | MMC60_Button_Gaming_B.stl | MMC60_Button_Gaming_X.stl |
| MMC60_Button_Gaming_Y.stl |

# 3D Printing Guide

The device was originally printed on a Bambu P1S using the Bambu Studio slicer, with a generic slicer profile for PLA filament.

## 3D Printing Summary

| Metrics | Single Unit |
| --- | --- |
| Total Print Time (hour min) | <PrintTime (e.g. 1h10m)> |
| Total Number of Components | <NumberOfComponents> |
| Typical Total Mass (g) | <TotalMassOfAllParts> |
| Typical Number of Print Setups | <NumberOfPrintSetups> |

## 3D Printing Settings

Note that the 3D printing material should be assumed to be PLA unless otherwise noted in the table below.

| Print File Name | Qty | Total Print Time (hr:min) | Mass (g) | Infill (%) | Support(Y/N) | Layer Height/ Nozzle Diameter(mm) | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| <Device_Name>_<Component>.stl | <Qty> | <Time> | <Mass> | <Infill> | <If supports are needed> | <layer Height/Nozzle diameter> |  |

## Post-Processing

Inspect the 3D printed parts for any printing defects, sharp edges, or burrs. Sharp edges and burrs can be removed with sanding or deburring tools.

<Add any additional processes that must be done after print, such as removing supports>

## Examples of Quality Prints

Compare your 3D prints to the images here. If there are significant differences, you may need to reprint the part.

<The table below can be expanded (adding rows), or compressed (removing columns) as needed, depending on the number of parts in the device>

| <Sub-Assembly Name> |  |  |
| --- | --- | --- |
| <Device_Name>_<Component>.stl | <Device_Name>_<Component>.stl | <Device_Name>_<Component>.stl |
| <INSERT IMAGE OF PART> | <INSERT IMAGE OF PART> | <INSERT IMAGE OF PART> |

# Maker Component List

<Fill in the table below. Continue the labeling convention for other added parts. Copy the table for another sub-assembly, if required, and label the parts as B<PartNumber>.>

| <Sub-Assembly Name> |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A01 | <Part Name> | QTY: <#> | A02 | <Part Name> | QTY: <#> | A03 | <Part Name> | QTY: <#> |
| <INSERT IMAGE OF PART> |  |  | <INSERT IMAGE OF PART> |  |  | <INSERT IMAGE OF PART> |  |  |

# Assembly Guide

### Part A: Required Components

<Copy and paste the first sub-assembly table from the Maker Component List section>.

### Part A: Required Tools and Supplies

<Add a bullet list of the required tools and supplies for this sub-assembly>

### Part A: Required Personal Protective Equipment (PPE)

<Add a bullet list of the required PPE for this sub-assembly>

# Testing

<Explain any testing the maker will have to complete to make sure the device is working properly>

# Troubleshooting

<Include any common errors / issues the maker may encounter and suggestions how to solve them.>